Free Flow Study

{{ project_name }}

{{ address }}

Tax Map {{ tax_map }}, Parcel {{ parcel }}

for

Scott Harbinson

9055 Meadowvale Court

Ellicott City MD, 21042

410-530-6925

Prepared by:

Mildenberg, Boender & Associates, Inc.

8318 Forrest Street

Ellicott City, MD 21043

(410) 997 0296

{{ author }}

{{ month_date }}

I hereby certify that these documents were prepared or approved by me and that I am a duly licensed Professional Engineer under the laws of the State of Maryland.

License no. {{ pe_license_no }}, Expiration date	 {{ pe_expiration }}

___________________________					___________________

{{ pe_full_name }}, P.E.						           Date: {{ date }}

85th Percentile Operating Speed Study

Mildenberg, Boender & Associates Inc. performed an 85th percentile operating speed determination at the access point to the above referenced parcel.

The proposed access point is located at {{ address_short }}. The survey was conducted between the hours of {{ start_time }} and {{ end_time }} on {{ date_long }}. The weather was {{ weather }} and {{ temp }}ºF. {{total_vehs}} vehicles were observed. The posted speed limit is {{ speed_limit }}mph on {{ road_name }}.

Raw Car Speeds (mph)

| {{ direction1 }} | {{ direction2 }} |
| --- | --- |
| 13 | 25 |
| 28 | 28 |
| 32 | 31 |
| 24 | 31 |
| 41 | 26 |
| 27 | 25 |
| 27 | 36 |
| 26 | 27 |
| 25 | 24 |
| 28 | 23 |
| 25 | 26 |
| 24 | 24 |
| 25 | 27 |
| 24 | 28 |
| 21 | 25 |
| 29 | 29 |
| 19 | 32 |
| 21 | 42 |
| 18 | 36 |
| 15 | 31 |
| 28 | 27 |
| 14 | 28 |
| 25 | 30 |
| 26 | 23 |
| 26 | 29 |
| 26 | 32 |
| 27 | 23 |
| 32 | 24 |
| 23 | 31 |
| 25 | 29 |
| 23 | 21 |
| 21 | 29 |
| 14 | 38 |
| 15 | 30 |
| 25 |  |
| 30 |  |

{{ direction1 }} Data

| Speed (mph) | Number of Cars Observed | Percentage of Cars Observed (%) | Cumulative Percentage of Cars Observed (%) |
| --- | --- | --- | --- |
| 13 | 1 | 2.8% | 2.8% |
| 14 | 2 | 5.6% | 8.3% |
| 15 | 2 | 5.6% | 13.9% |
| 18 | 1 | 2.8% | 16.7% |
| 19 | 1 | 2.8% | 19.4% |
| 21 | 3 | 8.3% | 27.8% |
| 23 | 2 | 5.6% | 33.3% |
| 24 | 3 | 8.3% | 41.7% |
| 25 | 6 | 16.7% | 58.3% |
| 26 | 4 | 11.1% | 69.4% |
| 27 | 3 | 8.3% | 77.8% |
| 28 | 3 | 8.3% | 86.1% |
| 29 | 1 | 2.8% | 88.9% |
| 30 | 1 | 2.8% | 91.7% |
| 32 | 2 | 5.6% | 97.2% |
| 41 | 1 | 2.8% | 100.0% |

{{ direction2 }} Data

| Speed (mph) | Number of Cars Observed | Percentage of Cars Observed (%) | Cumulative Percentage of Cars Observed (%) |
| --- | --- | --- | --- |
| 23 | 3 | 10.0% | 10.0% |
| 25 | 3 | 10.0% | 20.0% |
| 26 | 2 | 6.7% | 26.7% |
| 27 | 3 | 10.0% | 36.7% |
| 28 | 3 | 10.0% | 46.7% |
| 29 | 4 | 13.3% | 60.0% |
| 30 | 2 | 6.7% | 66.7% |
| 31 | 4 | 13.3% | 80.0% |
| 32 | 2 | 6.7% | 86.7% |
| 36 | 2 | 6.7% | 93.3% |
| 38 | 1 | 3.3% | 96.7% |
| 42 | 1 | 3.3% | 100.0% |

SUMMARY:

An off-peak spot speed study utilizing a radar was collected within the vicinity of the access point at the proposed easement.

The maximum {{ direction1 }} speed observed was {{ max_speed_dir1 }}mph and the minimum speed was {{ min_speed_dir1 }}mph.

The maximum {{ direction2 }} speed observed was {{ max_speed_dir2 }}mph and the minimum speed was {{ min_speed_dir2 }}mph.

The following are the operating speeds calculated at the access point to the project.

85th Percentile Speed

| road_name | Number of Vehicles Observed | 85th Percentile Speed (mph) |
| --- | --- | --- |
| direction1 | no_vehs_dir1 | speed_85_dir1 |
| direction2 | no_vehs_dir2 | Speed_85_dir2 |

Photos: